HIC Chromatography Column Sizing

Preliminary column sizing

| Validated DIR: 2-3-3-2-3-1 | Application: biopharmaceutical | Sized item: column |
| --- | --- | --- |

Sized-item concept

Reusable fixed sanitary in-place-packable HIC column; 400 L screening packed-resin volume, 30 cm selected bed height, and 1.303 m calculated unrounded internal diameter; final commercial-scale or custom vendor model TBD

Key specifications

| Item | Value | Unit |
| --- | --- | --- |
| Target protein load | 10 | kg/cycle |
| Selected screening packed-resin volume | 400 | L |
| Selected packed-bed height | 30 | cm |
| Calculated internal diameter | 1.303 | m |
| Selected loading residence time | 6 | min |
| Screening loading flow | 66.67 | L/min |
| Three-minute boundary flow | 133.33 | L/min |
| Preliminary minimum hardware rating | 5 | barg |
| Main process inlet and outlet | 2 (DN50) | in |
| Estimated installed envelope | 2.2 W × 2.2 D × 3.0 H | m |

Validated DIR `2-3-3-2-3-1`  |  System `HIC Chromatography`  |  Application `Biopharmaceutical`  |  Sized item `Column`  |  Document status `INITIAL DRAFT — pending process SME, resin supplier, and column vendor approval`

Design basis from DIR

| Requirement | Validated basis | Traceable status |
| --- | --- | --- |
| Target protein mass loaded | >1 to 10 kg/cycle; sizing value 10 kg/cycle | Provided; interpretation requires process SME confirmation |
| Usable resin loading capacity | >25 to 50 mg/mL packed resin | Validated DIR; treated as usable capacity without further utilization factor |
| Nominal packed-bed height | >25 to 40 cm | Validated DIR |
| Product-loading residence time | >3 to 6 min | Validated DIR |
| Column operating-pressure class | >3 to 5 bar | Validated DIR; rating definition requires vendor confirmation |
| Use and installation | Reusable fixed sanitary column with in-place packing and cleaning | Validated DIR |

The preliminary sizing uses the conservative capacity endpoint of 25 mg/mL, a selected 30 cm packed-bed height, and a selected 6 min loading residence time. The resulting 400 L resin requirement has no quantified capacity margin at exactly 25 mg/mL. Because the validated requirement is greater than 25 mg/mL, actual margin should be positive if that requirement is demonstrated, but it cannot be credited until an exact usable capacity is confirmed.

Sized-item concept and criteria

Reusable, fixed, sanitary, in-place-packable HIC column with approximately 400 L packed resin, 30 cm selected bed height, and 1.303 m calculated internal diameter. The diameter is an unrounded screening result rather than an available vendor model. Select the next suitable vendor configuration that provides at least 400 L at a DIR-compliant bed height and meets distributor, pressure, packing, drainage, cleanability, and maintenance requirements.

Preliminary construction criteria are 316L stainless-steel product-contact surfaces, hygienic drainable geometry, ASME BPE clamp-type interfaces, and EPDM or PTFE gaskets subject to chemical-compatibility review. The preliminary minimum hardware rating is 5 barg at design temperature; whether the validated pressure requirement applies to inlet pressure, differential pressure, MAWP, or another rating basis remains vendor-confirmation scope. No vacuum capability is assumed.

The calculated size is substantially larger than the cited Cytiva BPG catalog range of up to 450 mm ID and 121 L. The cited OPUS large-scale family has a catalog bed-height range of 10–30 cm and a 3 bar hardware maximum; those values are benchmarking information only and do not demonstrate compliance with the project pressure requirement. A commercial-scale or custom reusable column must therefore be confirmed without asserting a catalog model.

Capacity

| Item | Basis |
| --- | --- |
| Target load | 10 kg/cycle — Provided, interpretation unconfirmed. |
| DIR usable capacity range | >25 to 50 mg/mL — Validated DIR. |
| Calculated resin-volume range | 200 to 400 L — Calculated screening. |
| Selected screening resin volume | 400 L — Calculated from 25 mg/mL. |
| Selected packed-bed height | 30 cm — Screening assumption within the >25 to 40 cm DIR range. |
| Calculated area | 1.3333 m² — Calculated screening. |
| Calculated internal diameter | 1.3029 m — Calculated screening; vendor rounding required. |
| Nominal usable mass capacity at 25 mg/mL | 10.0 kg/cycle — Calculated; 0% margin at the conservative endpoint. |
| Nominal usable mass capacity at 50 mg/mL | 20.0 kg/cycle — Calculated range endpoint, not credited for selection. |
| Loading flow at 6 min residence time | 66.67 L/min — Calculated screening. |
| Flow corresponding to 3 min | 133.33 L/min — Calculated boundary value. Because the DIR requirement is greater than 3 min, the actual allowable loading flow is less than 133.33 L/min. |
| Loading superficial velocity at 30 cm and 6 min | 300 cm/h — Calculated screening. |

Processing time cannot be established because feed concentration, load volume, complete step recipe, cycle count, holds, transitions, and batch processing window are missing. Packed-bed pressure drop cannot be established because resin hydraulic properties and controlling high-salt and cleaning-fluid properties are unavailable.

Connections for the sized item

| Column-owned connection | Qty. | Preliminary size | Service and basis | Status |
| --- | --- | --- | --- | --- |
| Top process/mobile-phase inlet | 1 | 2 in (DN50) | Load, buffers, wash, elution, regeneration, CIP, and HETP/integrity-test liquid. At 133.33 L/min and 47.5 mm assumed bore, velocity is approximately 1.25 m/s. Estimated water-like local loss at K=2 is approximately 0.016 bar, excluding distributor and bed losses. | Calculated screening; vendor confirm |
| Bottom process/product outlet and low-point drain | 1 | 2 in (DN50) | Product, flow-through, process return, test discharge, and liquid drainage. Same preliminary hydraulic basis as the inlet; locate at the true low point without a retained pocket. | Calculated screening; vendor confirm |
| Top vent/air displacement | 1 | 1 in (DN25) | Air removal during filling and gas admission during gravity draining. Approximate ambient-air velocity is 5.8 m/s at 133.33 L/min displacement through a 22 mm assumed bore. | Calculated screening; vendor confirm |
| Upper flush pressure connection | 1 | 1.5 in (DN40) | Flush sanitary diaphragm seal for inlet-side pressure. Preliminary instrument range 0–6 barg. | Assumption; vendor confirm |
| Lower flush pressure connection | 1 | 1.5 in (DN40) | Flush sanitary diaphragm seal for outlet pressure and bed/distributor differential-pressure determination. | Assumption; vendor confirm |
| Upper resin-slurry charging/packing port | 1 | 3 in (DN80) | In-place resin charging. At 50% settled-resin fraction, 400 L packed resin corresponds to approximately 800 L slurry. A 200 L/min assumed transfer takes approximately 4 min and gives approximately 0.80 m/s in a 73 mm assumed bore. | Assumption; vendor confirm |
| Lower resin-slurry discharge/unpacking port | 1 | 3 in (DN80) | Resin removal and coarse solids drainage; provide flush, full-bore, pocket-free arrangement. | Assumption; vendor confirm |
| Outlet sample connection | 1 | 0.5 in (DN15) | Optional hygienic sample valve for qualification, elution, or cleaning verification. Omit if sampling is provided immediately downstream on the chromatography skid. | Optional assumption; vendor confirm |

All preliminary connections use ASME BPE hygienic clamp ferrules, 316L wetted parts, and EPDM or PTFE gaskets. Connection pressure boundaries are preliminarily rated for at least 5 barg at design temperature. Clamp derating, distributor losses, fitting losses, cyclic duty, gasket compatibility, and final skid interfaces require confirmation. No dedicated clean-steam, compressed-air, or separate buffer nozzle is included. The common inlet and outlet serve normal process, test, and CIP liquids.

Envelope / dimensions

Selected packed-bed geometry is approximately 1.303 m calculated ID × 0.30 m packed-bed height for 400 L — Calculated screening.

The supplied generic benchmark estimated an installed envelope of 2.2 m W × 2.2 m D × 3.0 m H — Estimate, with a recommended operating-space envelope of 4.2 m W × 4.2 m D × 4.5 m H — Estimate. That benchmark was generated from a 1.60 m ID × 0.20 m bed configuration providing approximately 402 L. The 20 cm benchmark bed height does not meet the validated DIR requirement of greater than 25 cm and is not the selected process geometry. Its envelope values are retained only as conservative facility-planning estimates and must be rebased by the selected column vendor.

Maintain approximately 1.0 m working clearance around the frame — Estimate and at least 1.5 m overhead maintenance clearance — Estimate for top-head, actuator, or distributor access. Confirm shipping splits, doorway and corridor access, lifting path, floor loading, support-point reactions, maximum operating weight, top-adapter travel, packing access, and maintenance withdrawal space.

Calculation table

| Calculation | Method | Numerical result | Unit | Status |
| --- | --- | --- | --- | --- |
| Target mass conversion | 10 kg × 1,000,000 mg/kg | 10,000,000 | mg/cycle | Calculated |
| Bed volume at 50 mg/mL | Mass / usable capacity | 200 | L | Calculated range endpoint |
| Bed volume at 25 mg/mL | Mass / usable capacity | 400 | L | Calculated governing screen |
| Selected bed volume | Conservative calculated requirement | 400 | L | Screening selection |
| Selected bed volume | Unit conversion | 0.400 | m³ | Calculated |
| Column area | Bed volume / 0.30 m | 1.3333 | m² | Calculated screening |
| Column internal diameter | √(4A/π) | 1.3029 | m | Calculated; unrounded |
| Nominal mass capacity | 400 L × 25 mg/mL | 10.0 | kg/cycle | Calculated |
| Capacity margin | (Capacity − load) / load | 0 | % | No credited margin at 25 mg/mL |
| Loading flow | 400 L / 6 min | 66.67 | L/min | Calculated screening |
| Three-minute boundary flow | 400 L / 3 min | 133.33 | L/min | Boundary only; actual maximum is lower |
| Loading superficial velocity | 0.30 m / 6 min | 300 | cm/h | Calculated screening |
| 2 in nozzle velocity | 133.33 L/min through 47.5 mm bore | 1.25 | m/s | Calculated screening |
| 2 in local nozzle loss | Kρv²/2 with K=2 and ρ=998 kg/m³ | 0.016 | bar | Water-like estimate; excludes bed/distributor |
| Vent air velocity | 133.33 L/min through 22 mm bore | 5.8 | m/s | Ambient displacement estimate |
| Resin slurry volume | 400 L / 0.50 settled fraction | 800 | L | Assumption-based estimate |
| Resin slurry transfer time | 800 L / 200 L/min | 4.0 | min | Assumption-based estimate |

Assumptions, exclusions, vendor confirmation

Assumptions

• The supplied 10 kg value represents target protein loaded per chromatography cycle.

• The product is the capacity-controlling species in a bind-elute HIC operation.

• The DIR capacity is already an effective usable design capacity; no additional 0.80 utilization factor is applied.

• The selected screening bed height is 30 cm and loading residence time is 6 min.

• No added resin-volume margin, packing allowance, scale-up factor, heel, or standard-size rounding is included.

• Water at 20 °C, density 998 kg/m³, is used only for the stated local nozzle-loss estimate and is not a controlling HIC hydraulic case.

• Fluids are preliminarily treated as incompressible, Newtonian, single-phase liquids. Salt-induced precipitation and suspended solids are excluded from the screening calculation.

• The column is not assumed to be vacuum rated; it must be vented during draining and cooling.

Missing process inputs — remain distinct from vendor confirmation

• Confirmation that 10 kg is target protein mass per cycle rather than batch mass or mixed-protein solids.

• HIC binding mode and identity of the capacity-controlling species.

• Conditioned-feed product and impurity concentrations and load volume per cycle.

• Batch mass, cycles per batch, permitted partial cycle, and batch processing window.

• Exact effective usable capacity or condition-specific DBC and utilization basis.

• Exact selected loading residence time and complete recipe for equilibration, load, wash, elution, regeneration, cleaning, sanitization, and storage.

• Minimum and peak flow for every step, acceptable pressure drop, and project line-sizing criteria.

• High-salt and cleaning-fluid composition, density, viscosity, and temperature range.

• Design temperature, pressure-design basis, credible pump shutoff or blocked-outlet pressure, and vacuum scenarios.

• Final sanitary standard, gasket specification, skid interface sizes, surface finish, and equipment battery limits.

Exclusions

• Chromatography skid, pumps, buffer vessels, facility utility headers, downstream sample systems, and interconnecting host piping.

• Final cycle-time assessment, pressure-drop calculation, resin lifetime, cleaning validation, structural design, code compliance, seismic design, floor-loading design, and qualification protocol.

• A vendor model commitment or guarantee of process performance.

Vendor confirmation

| Item | Basis |
| --- | --- |
| Resin supplier | resin identity, DBC or breakthrough curve at actual feed conditions, test residence time, breakthrough criterion, particle size, swelling, compressibility, allowable bed height, packing procedure, pressure-flow behavior, velocity limits, cleaning compatibility, and lifetime. |
| Column vendor | available diameter and bed-volume increments, at least 400 L packed volume at a DIR-compliant height, distributor uniformity and turndown, adjustable-adapter range, packing pressure, allowable bed differential pressure, hardware MAWP, rating definition, transient limits, vacuum rating, nozzle sizes, drainability, cleanability, materials, surface finish, and gasket compatibility. |
| Column vendor | certified installed and shipping envelope, maximum operating and shipping weights, center of gravity, support loads, access clearances, actuator services, lifting provisions, and maintenance withdrawal requirements. |
| Column and skid vendors jointly | final inlet/outlet interface sizes, pressure losses, pump shutoff protection, pressure-instrument locations, control philosophy, and allowable pressure at every operating and cleaning condition. |

This datasheet is an initial screening draft and is pending process SME, resin supplier, and column vendor approval.

| Item | Method/formula | Result | Unit | Basis |
| --- | --- | --- | --- | --- |
| Target protein mass conversion | Target mass multiplied by milligrams per kilogram; ={target_mass_kg}*{mg_per_kg} | ={target_mass_kg}*{mg_per_kg} | mg/cycle | Traceable conversion of the per-cycle target load |
| Required bed volume at conservative usable capacity | Target mass divided by conservative usable capacity; ={target_mass_mg}/{usable_capacity_low} | ={target_mass_mg}/{usable_capacity_low} | mL | Governing capacity volume before vendor-size rounding |
| Selected screening packed-bed volume | Conservative required volume converted from millilitres to litres; ={bed_volume_low_capacity_ml}/{ml_per_l} | ={bed_volume_low_capacity_ml}/{ml_per_l} | L | Must provide at least the calculated conservative capacity requirement |
| Required bed volume at upper usable capacity | Target mass divided by upper usable capacity and converted to litres; ={target_mass_mg}/{usable_capacity_high}/{ml_per_l} | ={target_mass_mg}/{usable_capacity_high}/{ml_per_l} | L | Defines the lower end of the DIR capacity-volume range |
| Selected packed-bed volume in cubic metres | Selected volume divided by litres per cubic metre; ={bed_volume_selected}/{l_per_m3} | ={bed_volume_selected}/{l_per_m3} | m³ | Unit conversion for geometry calculations |
| Required column cross-sectional area | Packed-bed volume divided by selected packed-bed height; ={bed_volume_m3}/{bed_height_selected} | ={bed_volume_m3}/{bed_height_selected} | m² | Selected bed height must satisfy the validated DIR range |
| Calculated column internal diameter | Square root of four times area divided by pi; =SQRT(4*{column_area}/PI()) | =SQRT(4*{column_area}/PI()) | m | Vendor-selected diameter must retain at least the required volume at a compliant bed height |
| Nominal usable mass capacity at conservative capacity | Bed volume multiplied by millilitres per litre and usable capacity, converted to kilograms; ={bed_volume_selected}*{ml_per_l}*{usable_capacity_low}/{mg_per_kg} | ={bed_volume_selected}*{ml_per_l}*{usable_capacity_low}/{mg_per_kg} | kg/cycle | Must be greater than or equal to target mass per cycle |
| Nominal conservative capacity margin | Nominal capacity minus target load, divided by target load; =({nominal_mass_capacity_kg}-{target_mass_kg})/{target_mass_kg}*100 | =({nominal_mass_capacity_kg}-{target_mass_kg})/{target_mass_kg}*100 | % | Project-required margin is not defined; zero margin is not suitable for design freeze |
| Screening loading flow | Selected packed-bed volume divided by selected residence time; ={bed_volume_selected}/{ebct_selected} | ={bed_volume_selected}/{ebct_selected} | L/min | Selected residence time must remain within the validated range and match the capacity basis |
| Flow at minimum residence-time boundary | Selected packed-bed volume divided by the DIR minimum-time boundary; ={bed_volume_selected}/{ebct_min_boundary} | ={bed_volume_selected}/{ebct_min_boundary} | L/min | Boundary only; actual loading flow must be lower because residence time must be greater than the boundary |
| Loading superficial velocity | Selected bed height divided by residence time and converted from metres per minute to centimetres per hour; ={bed_height_selected}/{ebct_selected}*100*60 | ={bed_height_selected}/{ebct_selected}*100*60 | cm/h | Must be within resin pressure-flow and performance limits |
| Boundary process flow conversion | Boundary flow converted from litres per minute to cubic metres per second; ={load_flow_boundary}/{l_per_m3}/60 | ={load_flow_boundary}/{l_per_m3}/60 | m³/s | Conversion used for connection screening |
| Main connection flow area | Pi times assumed bore squared divided by four; =PI()*{main_nozzle_id}^2/4 | =PI()*{main_nozzle_id}^2/4 | m² | Final bore must be based on the selected sanitary tube and ferrule schedule |
| Main inlet and outlet velocity | Boundary volumetric flow divided by connection flow area; ={boundary_flow_m3s}/{main_nozzle_area} | ={boundary_flow_m3s}/{main_nozzle_area} | m/s | Must not exceed the preliminary hygienic liquid-line velocity limit |
| Main connection local pressure loss | Local-loss coefficient times density times velocity squared divided by two and converted to bar; ={local_loss_k}*{reference_density}*{main_nozzle_velocity}^2/2/{pa_per_bar} | ={local_loss_k}*{reference_density}*{main_nozzle_velocity}^2/2/{pa_per_bar} | bar | Reference-only loss; distributor, frit, bed, valves, reducers, and external piping remain excluded |
| Vent flow area | Pi times assumed vent bore squared divided by four; =PI()*{vent_id}^2/4 | =PI()*{vent_id}^2/4 | m² | Vendor must verify gas-flow capacity and sanitary vent arrangement |
| Vent displacement-air velocity | Boundary displacement flow divided by vent area; ={boundary_flow_m3s}/{vent_area} | ={boundary_flow_m3s}/{vent_area} | m/s | Screening only; verify filter, valve, pressure, and vacuum cases |
| Estimated resin slurry volume | Selected packed-resin volume divided by settled-resin fraction; ={bed_volume_selected}/{settled_resin_fraction} | ={bed_volume_selected}/{settled_resin_fraction} | L | Replace with resin supplier and packing-procedure data |
| Estimated slurry transfer duration | Estimated slurry volume divided by assumed slurry flow; ={slurry_volume}/{slurry_flow} | ={slurry_volume}/{slurry_flow} | min | Must be compatible with resin shear, settling, hose, valve, and packing limits |